Valorization of Levulinic Acid through catalytic esterification with diols mediated by Zn(II)

Vincenzo Langellotti,1,2 Giuseppe Campanile,1 Maria Elena Cucciolito,1,2 Massimo Melchiorre,1,3 Vincenzo Russo,1 Francesco Ruffo1,2 and Roberto Esposito*,1,2

1 Dipartimento di Scienze Chimiche, Università degli Studi di Napoli Federico II, Complesso Universitario di Monte S. Angelo, Via Cintia 21, 80126 Napoli, Italy.

2 Consorzio Interuniversitario di Reattività Chimica e Catalisi (CIRCC), Via Celso Ulpiani 27, 70126 Bari, Italy.

3 ISusChem S.R.L., Piazza Carità 32, 80134 Napoli, Italy.

Email: vincenzo.langellotti@unina.it

Researchers have recently shifted their focus to converting and utilizing biomass in response to the increasing demand for environmentally friendly chemicals. Levulinic Acid (LA), a prominent product derived from lignocellulosic biomass, (1) has emerged as a well-established chemical platform.(2)

Among many fundamental compounds obtainable by LA, its esters (LE) are particularly interesting derivatives, finding applications as plasticizers, lubricants, and additives.(3)

Concurrently, there is a growing interest in the use of diol esters as bio-based lubricants.(4) In this study, we investigate the esterification of levulinic acid with polyols catalyzed by eco-friendly Zn(II), under mild and solvent-free conditions.

Scheme 1 - Esterification reaction of levulinic acid with polyols using Zn(II).

A screening of various Zn(II) species led to the identification of the best-performing catalyst, which was subsequently employed in the esterification of three different diols. Furthermore, we explored the possible formation of Zn(II) levulinate and its role in catalytic activity. Our study incorporates NMR methodology to assess the selectivity for mono- and di-esters and to monitor changes in the composition of the mixture during the reaction.

Acknowledgments: The authors are grateful to Italian Ministero dell’Istruzione dell’Università e della Ricerca for the financial support provided through the LEVANTE project “LEvulinic acid Valorization through Advanced Novel Technologies”, Progetti di Ricerca di Rilevante Interesse Nazionale -Bando 2020, Prot. 2020CZCJN7.

References

- Muranaka, Y.; Suzuki, T.; Sawanishi, H.; Hasegawa, I.; Mae, K. Ind. Eng. Chem. Res. 2014, 53, 11611.

- Pileidis, F. D.; Titirici, M. M. Chem. Sus. Chem. 2016, 9, 562G.

- Hayes, C.; Becer, C. R. Polym. Chem. 2020, 11, 4068–4077.

- Fortunato, M. E.; Taddeo, F.; Vitiello, R.; Turco, R.; Tesser, R.; Russo, V.; Di Serio, M. ACS Sus. Chem. Eng. 2023, 11, 33, 12406–12413.