DEHYDROGENATION OF FORMIC ACID CATALYZED BY IRIDIUM-BASED ORGANOMETALLIC COMPLEXES HAVING PYRIDINE-AMIDE LIGANDS WITH A SUGAR SUBSTITUENT

Vincenzo Langellotti,1,2 Alessandro Nataloni,3 Caterina Trotta,3 Immacolata Manco,1 Gabriel Menendez Rodriguez,3 Alceo Macchioni,3 Francesco Ruffo1,2

1 Dipartimento di Scienze Chimiche, Università degli Studi di Napoli Federico II,

Complesso Universitario di Monte S. Angelo, Via Cintia 21, 80126 Napoli, Italy.

2 Consorzio Interuniversitario di Reattività Chimica e Catalisi (CIRCC), Via Celso Ulpiani 27, 70126 Bari, Italy

3Dipartimento di Chimica, Biologia e Biotecnologie, Via Elce di Sotto, 06123 Perugia, Italy

A current valid strategy for hydrogen storage consists in the use of organic carriers that are non-toxic and liquid at room temperature (LOHCs).[1] These substances can be obtained from renewable energy sources through a hydrogenation reaction and can be easily transported to the desired place and be dehydrogenated to release molecular hydrogen. Among the most promising organic compounds, formic acid (FA) is of particular importance, because it can be dehydrogenated to produce molecular hydrogen and carbon dioxide according to the following reaction:

HCOOH CO2 + H2

Catalysis is essential for this reaction to occur. Complexes of Ru(II)[2] and Ir(III)[3] with the classical piano stool structure have proved to be particularly effective. In the context of our studies on iridium complexes containing ligands derived from sugars, we have recently[4] demonstrated the ability of complexes 1 and 2 (Figure 1) to hydrogenate enantioselectively lactic acid and its derivatives. In this study they were tested in the dehydrogenation of formic acid in aqueous solution. The catalytic activity has been probed by systematic kinetic investigations, evaluating effect of pH, temperature, and the concentration of the catalyst and of the buffer solution. Finally, targeted experiments allowed to bring out the high durability of these catalysts.

Figure 1: Structure of complexes 1 (R= H) and 2 (R= OAc)

Acknowledgments: This study was carried out within the Agritech National Research Center and received funding from the European Union Next-GenerationEU (PIANO NAZIONALE DI RIPRESA E RESILIENZA (PNRR) MISSIONE 4 COMPONENTE 2, INVESTIMENTO 1.4 - D.D. 1032 17/06/2022, CN00000022).

[1] L. Fan, et al., Energy Reports, 7 (2021), 8421.

[2] C. Guan et al., Inorg. Chem., 56 (2017), 438.

[3] W.-H. Wang et al., ChemSusChem, 7 (2014), 1976.

[4] L. Tensi et al., Organometallics, 42 (2023), 157